## Wallace Community Service District Annual Water Quality Report

Newsletter											         July 2011

| Este informe contiene información muy importante sobre su agua potable. Tradúzcalo ó hable con alguien que lo entienda bien. Board of Directors President Patsy Bailey Vice-President Larry Howen Directors Cathryn Jackson David Reyner Jerry Zedlitz Board Secretary (Volunteer) Jackie Neill Staff GM Bill Perley - CCWD Deputy Director of Operations Teresa Tanaka - CCWD Utility Operator(s) John Brown Jon Ingebretsen Sam Singh Inside this Issue: Water Quality Data Page 2 Water Quality Data Page 3 Definitions and Page 3 Abbreviations Water Use Comparison Page 4; We are pleased to present to you this year’s Annual Water Quality Report. This report is designed to inform you about the quality of water and services we deliver to you each and every day. Our constant goal is to provide you with a safe and dependable supply of drinking water. This report shows the results of our monitoring for the period of January 1 – December 31, 2010. Wallace’s drinking water comes from one active well (Well #2) located within the District’s service area and one standby well (Well #3); which is currently being certified for full time service. Contaminants that may be present in source water include: - Microbial contaminants, such as viruses and bacteria that may come from sewage treatment plants, septic systems, agricultural livestock operations and wildlife. - Inorganic contaminants, such as salts and metals that can be naturally-occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining or farming. - Pesticides and herbicides that may come from a variety of sources such as agriculture, urban stormwater runoff and residential uses. - Organic chemical contaminants, including synthetic and volatile organic chemicals that are byproducts of industrial processes and petroleum production, and can also come from gas stations, urban stormwater runoff, agricultural application and septic systems. - Radioactive contaminants that can be naturally-occurring or be the result of oil and gas production and mining activities. In order to ensure that tap water is safe to drink, USEPA and the state Department of Public Health (Department) prescribe regulations that limit the amount of certain contaminants in water provided by public water systems. Department regulations also establish limits for contaminants in bottled water that provide the same protection for public health. Tables 1, 2, 3, 4, and 5 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent. The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk. The Department allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently. Some of the data, though representative of the water quality, are more than one year old. Any violation of a MCL or AL is marked with an asterisk. Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily indicate that the water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the USEPA’s Safe Drinking Water Hotline (1-800-426-4791). Some people may be more vulnerable to contaminants in drinking water than the general population. Immuno-compromised persons such as those with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. USEPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Crystospordium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).. DRINKING WATER SOURCE ASSESSMENT INFORMATION An assessment of the drinking water sources for Wallace Community Services District was completed in March 2001for the one active and one standby groundwater sources , (Wells 2 and 3). The sources are considered most vulnerable to the following activities: recreational areas, surface water and other water supply wells. A copy of the assessment can be obtained by contacting WCSD at (209) 763-2882 or from the Stockton Office of the Department of Public Health at (209) 948-7696 |
| --- |
|  | Table 1 - sampling results showing the detection of coliform bacteria |  |  |  |  |  |  |  |
|  | Microbiological Contaminants | Highest No. of detections | Violations In a month | MCL |  | MCLG | Typical Source of Bacteria |  |
|  | Total Coliform Bacteria | (In a mo.) 0 | 0 | More than 1 sample in a month with a detection |  | 0 | Naturally present in the environment |  |
|  | Fecal Coliform or E. coli | (In the year) 0 | 0 | A routine sample and a repeat sample detect total coliform and either sample also detects fecal coliform or E. coli |  | 0 | Human and animal fecal waste |  |
|  | Table 2 - sampling results showing the detection of Lead and copper |  |  |  |  |  |  |  |
|  | Lead and Copper (to be completed only if there was a detection of lead or copper in the last sample set) | No. of samples collected | 90th percentile level detected | No. Sites exceeding AL | AL | MCLG | Typical Source of Contaminant |  |
|  | Lead (ppb) 2010 | 5 | 6.0 ug/L | 0 | 15 | 0.2 | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |  |
|  | Copper (ppm) 2010 | 5 | 0.22 mg/L | 0 | 1.3 | 0.17 | Internal corrosion of household water plumbing systems; erosion of natural deposits; leaching from wood preservatives |  |
|  | TAble 3 - sampling results for sodium and hardness |  |  |  |  |  |  |  |
|  | Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |  |
|  | Sodium (ppm) Well 2 | 6/09/10 | 20 | N/A | none | none | Generally found in ground and surface water |  |
|  | Hardness (ppm) Well 2 | 6/09/10 | 56 | N/A | none | none | Generally found in ground and surface water |  |
|  | TAble 4 - detection of contaminants with a Primary Drinking Water Standard |  |  |  |  |  |  |  |
|  | Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |  |
|  | TTHM (ppb) Total Trihalomethanes | 7/14/10 | 3.7 | N/A | 80 | N/A | By-product of drinking water disinfection |  |
|  | HAA5’s (ppb) Haloacetic Acids | 7/14/10 | ND | N/A | 60 | N/A | By-product of drinking water disinfection |  |
|  | TAble 5 - detection of contaminants with a Secondary Drinking Water Standard |  |  |  |  |  |  |  |
|  | Chemical or Constituent (and reporting units) | Sample Date | Level Detected | Range of Detections | MCL | PHG (MCLG) | Typical Source of Contaminant |  |
|  | Aluminum Well 2 | 04-02-09 | 46 | N/A | 1000 ppb | N/A | Erosion of natural deposits; residue from some surface water treatment processes |  |
|  | Iron (ppb) Well #2 Treated | 2010 2010 | 1742 52 | 600 - 2000 ND - 139 | 300 ppb | N/A | Leaching from natural deposits; industrial waste |  |
|  | Manganese (ppb) Well #2 Treated | 2010 2010 | 530 159 | 410 - 640 <5.0 – 46 | 50 ppb | N/A N/A | Leaching from natural deposits |  |
|  | Chloride - Well 2 | 06/09/10 | 9.6 |  | 500 ppm | N/A | Runoff/leaching from natural deposits; seawater influence |  |
|  | Sulfate – Well 2 | 06/09/10 | 5.8 |  | 500 ppm | N/A | Runoff/leaching from natural deposits; industrial wastes |  |

| TERMS USED IN THIS REPORT: |  |  |  |
| --- | --- | --- | --- |
| Maximum Contaminant Level (MCL): The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. Primary Drinking Water Standards (PDWS): MCLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. Secondary Drinking Water Standards (SDWS): MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. Maximum Residual Disinfectant Level (MRDL): The level of a disinfectant added for water treatment that may not be exceeded at the consumer’s tap. Maximum Residual Disinfectant Level Goal (MRDLG): The level of a disinfectant below which there is no known or expected risk to health. MRDLGs are set by the U.S. Environmental Protection Agency. |  | Public Health Goal (PHG): The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (USEPA). Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water. Regulatory Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow. Variances and Exemptions: Department permission to exceed an MCL or not comply with a treatment technique under certain conditions. ND: not detectable at testing limit ppm: parts per million or milligrams per liter (mg/L) ppb: parts per billion or micrograms per liter (ug/L) ppt: parts per trillion or nanograms per liter (ng/L) pCi/L: picocuries per liter (a measure of radiation) |  |
| WALLACE COMMUNITY SERVICES DIST. P.O. BOX 398 100 Wallace Lake Drive Wallace, CA 95254 PHONE: (209) 763-2882 FAX: (209) 763-2924 E-MAIL: jackieneill@comcast.net OFFICE HOURS: Office is not manned on a daily basis, call the number above and leave a message. ##### PUBLIC WELCOME Monthly Board Meetings 3rd Thursday of every month @ 7:00 p.m. at the Historic Wallace School House on 799 Ward Ave Quality on Tap Our Commitment Our Profession | Return Service Requested Fact or Fiction. - About half the water treated by public water systems is used for cooking. (Fiction: Actually the amount of water used for cooking and drinking is less than 1% of the total water produced!) - A person can live about a month without food, but only about a week without water. (Fact: Dehydration symptoms generally become noticeable after only 2% of one’s normal water volume has been lost.) - The first water pipes in the U.S. were made of cast iron. (Fiction: The first water pipes were actually made of fire-charred bored logs.) - The world’s first municipal water filtration plant was opened in the United States. (Fiction: The first plant was actually opened in Paisley, Scotland, in 1832.) - A person must consume a half-gallon of water daily to live healthily. (Fact: A person should drink at least 64 ounces, or 8 cups, of water each day.) - One gallon of gasoline poured into a lake can contaminate approximately 750,000 gallons of water. (Fact) |